深圳市驰普电子科技有限公司

CHEAPE TECHNOLOGY INTERNATIONAL LTD

Acknowledgment

| Customer Name |  | Chi Pu electronic |  |  |  | Productmodel | D34CELL 4 strings |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Delivery date |  | 2012-10-25 |  |  |  | To configure | 8254AAF+4407*8+10Milliohms |
| Revision |  |  | Page | 03 | Revision |  | A1 |
| Approved |  |  |  | TO examine |  |  | Elaboration |
| Material Number |  |  |  |  |  |  |  |
| Customer confirmation column |  |  |  |  |  |  |  |
| Confirmation opinions： Signature： Date： |  |  |  |  |  |  |  |
| Special Note: - Customer receive samples promptly after test,the test results will be back to my company FAX:0755-61186524, to facilitate the company to arrange for the project follow-up work, 5 days without any reply,the company for customers to test qualified default,the project ended normally. - Customer test qualified,please indicate the product customer feedback column names and product code,and stamped signature confirmation,otherwise please test unqualified column indicate te problem,put forward the improvement proposal. - My company received after customer signed original and enclose the product description functional description,in order to receive orders. |  |  |  |  |  |  |  |

# Contents

1 Outline

This specification is suitable 4-serial-cell Lithium ion Battery Protection circuit manufactured by CTECHI TE CO.,LTD

2 Application

- Lithium-ion rechargeable battery packs

- Lithium-ion polymer battery packs

3 Electrical characteristics

| ### Item | Content | Criterion |  |
| --- | --- | --- | --- |
| Over charge Protection | Monomer overcharge protection voltage | 4.35±0.025V |  |
|  | Monomer overcharge release voltage | 4.15±0.05V |  |
| ### Charge protection | Rated charging current | ≤4A |  |
|  | Charge current protection | / |  |
| Over discharge protection | Single discharge protection voltage | 2.40±0.08V |  |
|  | Monomer overdischarge release voltage | 3.0±0.1V |  |
|  | Nominal discharge current | ≤8A |  |
|  | Discharge current protection | 20±3A |  |
| ### Short protection | There is no short-circuit protection | Yes |  |
|  | Protection of short circuit current | / |  |
|  | Short circuit protection restoration condition | Disconnect the load |  |
| Temperature protection | Thermal protection temperature e | / |  |
|  | Recovery temperature/time | / |  |
| Balanced | Balanced voltage | / |  |
|  | Equilibrium current | / |  |
| Interior resistance | P+,P-conduction resistance | ≤40mΩ |  |
| Current consumption | Self power consumption | ≤35μA |  |
| (L*W*H) Size | 72*28*1.0mm Tolerance：±0.1MM |  |  |

4 Wiring diagram

Prot description：

1、B+：Connection fourth section of battery core anode

2、VH：Connection third section of battery core anode

3、VM：Connection second section of battery core anode

4、VL：Connect first battery core anode

5、B-：Connect the negative battery cell section

6、P+：Connecting the output charge / discharge anode

7、P-：Connecting the output charge / discharge cathode